# I. BẢNG CHIA IP TOÀN HỆ THỐNG

## 1. Trụ sở Hồ Chí Minh (HQ)

| Khu vực | Size | Address | Subnet mask | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- |
| Floor 1 – KT | 62 | 192.168.0.0/26 | 255.255.255.192 | 192.168.0.63 | 192.168.0.1 |
| Floor 2 – IT | 62 | 192.168.0.64/26 | 255.255.255.192 | 192.168.0.127 | 192.168.0.65 |
| Floor 3 – QLRR | 62 | 192.168.0.128/26 | 255.255.255.192 | 192.168.0.191 | 192.168.0.129 |
| Floor 4 – Server Room | 30 | 192.168.0.192/27 | 255.255.255.224 | 192.168.0.223 | 192.168.0.193 |
| Wi-Fi Khách (HQ) | 254 | 192.168.1.0/24 | 255.255.255.0 | 192.168.1.255 | 192.168.1.1 |
| p2p R1↔L3SW_1 | 2 | 192.168.253.0/30 | 255.255.255.252 | 192.168.253.3 | – |
| p2p R2↔L3SW_2 | 2 | 192.168.253.4/30 | 255.255.255.252 | 192.168.253.7 | – |
| p2p R1↔R2 | 2 | 192.168.253.8/30 | 255.255.255.252 | 192.168.253.11 |  |
| p2p L3SW_1↔L3SW_2 | 2 | 192.168.254.0/30 | 255.255.255.252 | 192.168.254.3 | – |

## 2. Chi nhánh Đà Nẵng

| Khu vực | Size | Address | Subnet mask | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- |
| Nhân viên | 62 | 192.168.3.0/26 | 255.255.255.192 | 192.168.3.63 | 192.168.3.1 |
| p2p DN_L3_SW1 ↔ DN_L3_SW2 | 2 | 192.168.252.0/30 | 255.255.255.252 | 192.168.252.3 | – |
| p2p DN_R1 ↔ DN_L3_SW1 | 2 | 192.168.251.0/30 | 255.255.255.252 | 192.168.251.3 | – |
| p2p DN_R2 ↔ DN_L3_SW2 | 2 | 192.168.251.4/30 | 255.255.255.252 | 192.168.251.7 | – |
| p2p DN_R1↔DN_R2 | 2 | 192.168.251.8/30 | 255.255.255.252 | 192.168.251.11 | – |

## 3. Chi nhánh Hà Nội

| Khu vực | Size | Address | Subnet mask | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- |
| Nhân viên | 62 | 192.168.2.0/26 | 255.255.255.192 | 192.168.2.63 | 192.168.2.1 |
| p2p HN_L3_SW1 ↔ HN_L3_SW2 | 2 | 192.168.250.0/30 | 255.255.255.252 | 192.168.250.3 | – |
| p2p HN_R1 ↔ HN_L3_SW1 | 2 | 192.168.249.0/30 | 255.255.255.252 | 192.168.249.3 | – |
| p2p HN_R2 ↔ HN_L3_SW2 | 2 | 192.168.249.4/30 | 255.255.255.252 | 192.168.249.7 | – |
| p2p HN_R1↔HN_R2 | 2 | 192.168.249.8/30 | 255.255.255.252 | 192.168.249.11 | – |

## 5. Kết nối Internet

| Liên kết | Subnet (/30) | Địa chỉ ISP | Địa chỉ Router (site) | Broadcast |
| --- | --- | --- | --- | --- |
| HQ — R1 | 203.0.113.0/30 | 203.0.113.1 | 203.0.113.2 (HQ_R1) | 203.0.113.3 |
| HQ — R2 | 203.0.113.4/30 | 203.0.113.5 | 203.0.113.6 (HQ_R2) | 203.0.113.7 |
| HN — R1 | 203.0.113.8/30 | 203.0.113.9 | 203.0.113.10 (HN_R1) | 203.0.113.11 |
| HN — R2 | 203.0.113.12/30 | 203.0.113.13 | 203.0.113.14 (HN_R2) | 203.0.113.15 |
| DN — R1 | 203.0.113.16/30 | 203.0.113.17 | 203.0.113.18 (DN_R1) | 203.0.113.19 |
| DN — R2 | 203.0.113.20/30 | 203.0.113.21 | 203.0.113.22 (DN_R2) | 203.0.113.23 |

# II. BẢNG IP THIẾT BỊ

## 1. Trụ sở Hồ Chí Minh (HQ)

| Device | Interface | IP address | Subnet Mask | Default GW |
| --- | --- | --- | --- | --- |
| R1-HCM | f0/0 → L3SW_1 | 192.168.253.1 | 255.255.255.252 | – |
|  | f1/0 → R2 | 192.168.253.9 | 255.255.255.252 | – |
|  | f2/0 → Internet | 203.0.113.2 | 255.255.255.252 | – |
|  | Tunnel 0 | 192.168.200.1 | 255.255.255.252 | – |
|  | Tunnel 1 | 192.168.201.1 | 255.255.255.252 | – |
|  | Tunnel 2 | 192.168.202.1 | 255.255.255.252 | – |
|  | Tunnel 3 | 192.168.203.1 | 255.255.255.252 | – |
| R2-HCM | f0/0 → L3SW_2 | 192.168.253.5 | 255.255.255.252 | – |
|  | f1/0 → R1 | 192.168.253.10 | 255.255.255.252 | – |
|  | f2/0 → Internet | 203.0.113.6 | 255.255.255.252 | – |
|  | Tunnel 0 | 192.168.200.1 | 255.255.255.252 | – |
|  | Tunnel 1 | 192.168.201.1 | 255.255.255.252 | – |
|  | Tunnel 2 | 192.168.202.1 | 255.255.255.252 | – |
|  | Tunnel 3 | 192.168.203.1 | 255.255.255.252 | – |
| L3SW_1 | f1/0 (uplink R1) | 192.168.253.2 | 255.255.255.252 | – |
|  | vlan 10 | 192.168.0.1 | 255.255.255.192 | – |
|  | vlan 20 | 192.168.0.65 | 255.255.255.192 | – |
|  | vlan 30 | 192.168.0.129 | 255.255.255.192 | – |
|  | vlan 40 | 192.168.0.193 | 255.255.255.224 | - |
| L3SW_2 | f1/0 (uplink R2) | 192.168.253.6 | 255.255.255.252 | – |
|  | vlan 10 | 192.168.0.1 | 255.255.255.192 | – |
|  | vlan 20 | 192.168.0.65 | 255.255.255.192 | – |
|  | vlan 30 | 192.168.0.129 | 255.255.255.192 | – |
|  | vlan 40 | 192.168.0.193 | 255.255.255.224 | - |
| Laptop_KT | Wi-Fi | DHCP | – | 192.168.0.1 |
| KT_PC | e0/0 | DHCP | – | 192.168.0.1 |
| Laptop_IT | Wi-Fi | DHCP | – | 192.168.0.65 |
| PC_IT | e0/0 | DHCP | – | 192.168.0.65 |
| Laptop_QLRR | Wi-Fi | DHCP | – | 192.168.0.129 |
| PC_QLRR | e0/0 | DHCP | – | 192.168.0.129 |

## 2. Chi nhánh Hà Nội

| Device | Interface | IP address | Subnet Mask | Default GW |
| --- | --- | --- | --- | --- |
| HN_R1 | f0/0 | 192.168.249.1 | 255.255.255.252 | – |
|  | f1/0 | 192.168.249.9 | 255.255.255.252 | – |
|  | f2/0 | 203.0.113.10 | 255.255.255.252 | – |
|  | Tunnel 0 | 192.168.200.2 | 255.255.255.252 | – |
|  | Tunnel 1 | 192.168.201.1 | 255.255.255.252 | – |
|  | Tunnel 2 | 192.168.204.1 | 255.255.255.252 | – |
|  | Tunnel 3 | 192.168.205.1 | 255.255.255.252 | – |
| HN_R2 | f0/0 | 192.168.249.5 | 255.255.255.252 | – |
|  | f1/0 | 192.168.249.10 | 255.255.255.252 | – |
|  | f2/0 | 203.0.113.14 | 255.255.255.252 | – |
|  | Tunnel 0 | 192.168.200.2 | 255.255.255.252 | – |
|  | Tunnel 1 | 192.168.201.2 | 255.255.255.252 | – |
|  | Tunnel 2 | 192.168.206.2 | 255.255.255.252 | – |
|  | Tunnel 3 | 192.168.207.1 | 255.255.255.252 | – |
| HN_L3_SW1 | f0/4 | 192.168.2.2 | 255.255.255.192 | – |
|  | f0/1 | 192.168.2.66 | 255.255.255.192 | – |
|  | f1/0 | 192.168.250.1 | 255.255.255.252 | – |
|  | f2/0 | 192.168.249.2 | 255.255.255.252 | – |
| HN_L3_SW2 | f0/4 | 192.168.2.3 | 255.255.255.192 | – |
|  | f0/1 | 192.168.2.67 | 255.255.255.192 | – |
|  | f1/0 | 192.168.250.2 | 255.255.255.252 | – |
|  | f2/0 | 192.168.249.6 | 255.255.255.252 | – |
| HN_PC1 | e0/0 | DHCP | – | 192.168.2.1 |
| HN_PC2 | e0/0 | DHCP | – | 192.168.2.65 |

## 3. Chi nhánh Đà Nẵng

| Device | Interface | IP address | Subnet Mask | Default GW |
| --- | --- | --- | --- | --- |
| DN_R1 | f0/0 | 192.168.251.1 | 255.255.255.252 | – |
|  | f1/0 | 192.168.251.9 | 255.255.255.252 | – |
|  | f2/0 | 203.0.113.18 | 255.255.255.252 | – |
|  | Tunnel 0 | 192.168.204.2 | 255.255.255.252 | – |
|  | Tunnel 1 | 192.168.206.2 | 255.255.255.252 | – |
|  | Tunnel 2 | 192.168.202.2 | 255.255.255.252 | – |
|  | Tunnel 3 | 192.168.203.2 | 255.255.255.252 | – |
|  | Tunnel 4 | 192.168.203.1 | 255.255.255.252 | – |
| DN_R2 | f0/0 | 192.168.251.5 | 255.255.255.252 | – |
|  | f1/0 | 192.168.251.10 | 255.255.255.252 | – |
|  | f2/0 | 203.0.113.22 | 255.255.255.252 | – |
|  | Tunnel 0 | 192.168.207.2 | 255.255.255.252 | – |
|  | Tunnel 1 | 192.168.205.2 | 255.255.255.252 | – |
|  | Tunnel 2 | 192.168.202.2 | 255.255.255.252 | – |
|  | Tunnel 3 | 192.168.203.2 | 255.255.255.252 | – |
|  | Tunnel 4 | 192.168.200.2 | 255.255.255.252 | – |
| DN_L3_SW1 | f0/4 | 192.168.3.2 | 255.255.255.192 | – |
|  | f0/1 | 192.168.3.66 | 255.255.255.192 | – |
|  | f1/0 | 192.168.252.1 | 255.255.255.252 | – |
|  | f2/0 | 192.168.251.2 | 255.255.255.252 | – |
| DN_L3_SW2 | f0/4 | 192.168.3.3 | 255.255.255.192 | – |
|  | f0/1 | 192.168.3.67 | 255.255.255.192 | – |
|  | f1/0 | 192.168.252.2 | 255.255.255.252 | – |
|  | f2/0 | 192.168.251.6 | 255.255.255.252 | – |
| DN_PC1 | e0/0 | DHCP | – | 192.168.3.1 |
| DN_PC2 | e0/0 | DHCP | – | 192.168.3.65 |

# III. Bảng chia VLAN, cấu hình SVI và VPN

## 1. Chi nhánh Thành phố Hồ Chí Minh

| VLAN ID | Tên VLAN | Subnet | Gateway (Virtual gateway) | Active Gateway | Standby Gateway | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | VLAN_KT | 192.168.0.0/26 | 192.168.0.1 | 192.168.0.2 | 192.168.0.3 | Tầng 1 |
| 20 | VLAN_IT | 192.168.0.64/26 | 192.168.0.65 | 192.168.0.67 | 192.168.0.66 | Tầng 2 |
| 30 | VLAN_QLRR | 192.168.0.128/26 | 192.168.0.129 | 192.168.0.130 | 192.168.0.131 | Tầng 3 |
| 40 | VLAN_SERVER | 192.168.0.192/27 | --- | --- | --- | Tầng 4 |
| 50 | VLAN _Guest_WIFI | 192.168.1.0/24 | --- | --- | --- | Wifi |

## 2. VPN Site-to-Site

| Link | Device A | Device B | IP A | IP B | Subnet mask |
| --- | --- | --- | --- | --- | --- |
| HQ ↔ Hà Nội | R1-HCM (Tunnel 0) | R1-HN (Tunnel 0) | 192.168.200.1 | 192.168.200.2 | 255.255.255.252 |
|  | R1-HCM (Tunnel 1) | R2-HN (Tunnel 1) | 192.168.201.1 | 192.168.201.2 | 255.255.255.252 |
|  | R2-HCM (Tunnel 0) | R2-HN (Tunnel 0) | 192.168.200.1 | 192.168.200.2 | 255.255.255.252 |
|  | R2-HCM (Tunnel 1) | R1-HN (Tunnel 1) | 192.168.201.1 | 192.168.201.2 | 255.255.255.252 |
| HQ ↔ Đà Nẵng | R1-HCM (Tunnel 2) | R1-ĐN (Tunnel 2) | 192.168.202.1 | 192.168.202.2 | 255.255.255.252 |
|  | R1-HCM (Tunnel 3) | R2-ĐN (Tunnel 3) | 192.168.203.1 | 192.168.203.2 | 255.255.255.252 |
|  | R2-HCM (Tunnel 2) | R2-ĐN (Tunnel 2) | 192.168.202.1 | 192.168.202.2 | 255.255.255.252 |
|  | R2-HCM (Tunnel 3) | R1-ĐN (Tunnel 3) | 192.168.203.1 | 192.168.203.2 | 255.255.255.252 |
| Hà Nội ↔ Đà Nẵng (backup) | R1-HN (Tunnel 2) | R1-ĐN (Tunnel 0) | 192.168.204.1 | 192.168.204.2 | 255.255.255.252 |
|  | R1-HN (Tunnel 3) | R2-ĐN (Tunnel 1) | 192.168.205.1 | 192.168.205.2 | 255.255.255.252 |
|  | R2-HN (Tunnel 2) | R1-ĐN (Tunnel 1) | 192.168.206.1 | 192.168.206.2 | 255.255.255.252 |
|  | R2-HN (Tunnel 3) | R2-ĐN (Tunnel 0) | 192.168.207.1 | 192.168.207.1 | 255.255.255.252 |

# IV. ACL

- VLAN10 (Kế toán)

- Chỉ được:

  - Truy cập server nội bộ (ví dụ: 10.10.10.0/24 nếu bạn có)

  - Ra internet qua firewall/router

- Không được:

  - Truy cập VLAN20, VLAN30

  - SSH/SNMP/OSPF đến thiết bị mạng

- VLAN20 (Quản lý rủi ro)

- Tương tự VLAN10 nhưng có thể cho phép đọc log từ server phân tích rủi ro.

- VLAN30 (IT)

- Toàn quyền:

  - SSH vào switch, router

  - SNMP/Zabbix

  - Ping các mạng khác

  - Truy cập toàn bộ VLAN

ACL cho VLAN10 – Kế Toán

ip access-list extended VLAN10_IN ! Cho phép truy cập server nội bộ (nếu có) permit ip 192.168.0.0 0.0.0.63 10.10.10.0 0.0.0.255 ! Cho phép ra Internet (default route) permit ip 192.168.0.0 0.0.0.63 any ! Chặn truy cập VLAN20, VLAN30 deny ip 192.168.0.0 0.0.0.63 192.168.0.64 0.0.0.63 deny ip 192.168.0.0 0.0.0.63 192.168.0.128 0.0.0.63 ! Chặn truy cập thiết bị mạng deny tcp 192.168.0.0 0.0.0.63 any eq 22 deny udp 192.168.0.0 0.0.0.63 any eq snmp deny ospf 192.168.0.0 0.0.0.63 any ! Mặc định deny toàn bộ còn lại (nguyên tắc Zero Trust)

ACL cho VLAN20 – Quản lý rủi ro

ip access-list extended VLAN20_IN ! Cho phép truy cập server nội bộ phân tích rủi ro permit ip 192.168.0.64 0.0.0.63 10.10.20.0 0.0.0.255 ! Cho phép ra Internet permit ip 192.168.0.64 0.0.0.63 any ! Chặn truy cập VLAN10 và VLAN30 deny ip 192.168.0.64 0.0.0.63 192.168.0.0 0.0.0.63 deny ip 192.168.0.64 0.0.0.63 192.168.0.128 0.0.0.63 ! Chặn truy cập thiết bị mạng deny tcp 192.168.0.64 0.0.0.63 any eq 22 deny udp 192.168.0.64 0.0.0.63 any eq snmp deny ospf 192.168.0.64 0.0.0.63 any

ACL cho VLAN30 – IT

ip access-list extended VLAN30_IN ! IT full control permit ip 192.168.0.128 0.0.0.63 any

Trên L3SW_1:

interface vlan 10 ip access-group VLAN10_IN in interface vlan 20 ip access-group VLAN20_IN in interface vlan 30 ip access-group VLAN30_IN in

Trên L3SW_2

interface vlan 10 ip access-group VLAN10_IN in interface vlan 20 ip access-group VLAN20_IN in interface vlan 30 ip access-group VLAN30_IN in